13cm Feed Dimensions

(Scaled from the RA3AQ design on 1296MHz)

- The whole feed is built from 5mm thick copper sheet.

- The septum MUST be bonded to the back short

- “Seam soldering”  produces a more consistent bond b/w the various elements

- 3/16” brass tuning screws were used as in the 1296 MHz OK1DFC septum feed

- With feed probe dimensions and positioning shown and with the tuning screws I achieved 22dB Port-Port isolation with a RL of 24dB on each port.

- Some compromise is required to achieve a balance b/w the ports for both RL and Port-Port Isolation

73 VK3NX