VK3NX – DF1OI  24 GHz  CW

1st  VK-DL 24GHz  CW  QSO

16 194.8 km

Date:  10/10/2014

Report:   DF1OI Received at  VK3NX   =  O

VK3NX  Received at DF1OI= M

Equipment at DF1OI

- 2.4 m  offset fed

- Pwr  48 W

- At time of QSO…  Moon noise = 1.3-1.5dB  @  DF1OI

- Tracking  moon noise  and  12 bit encoders

Equipment VK3NX:

- 2.4 m  Solid dish  (Andrews)

- 11.5 W Kuhne SSPA  (10 W at feed)  short  WR42  flexible W/G  to  W/G Switch (~0.5dB loss)

- Feed:  Super VE4MA  choke flange   with  Circular to rectangular transition (WR42)

- W/G switch:  “EVENTS HORIZON”

- LNA @ feed

  - Kuhne 1.5dB NF  27 db gain  +  DB6NT design (homebrew)  NF= Max 2.0dB  Gain =  11.5dB

  - Total  gain is 38.5dB in “feed box”

- LMR240  with Radiall 18GHz connectors  for  short  run  on Tx  and Rx  from “feed box” to  transverter

- Transverter = Thales  26 GHz  units  running  1296 MHz  IF.  Then  down-converted to  144MHz IF.   IF  driver  IC-746.  ( With  500 Hz  filter  used for CW  contact)

- Kuhne  24 GHz  W/G  filters  on  Rx and Tx  ports of Thales  unit.  (Absolutely necessary due to very   poor image rejection ~4dB only  with 1296 MHz  IF)

Perfomance of Rx  at  VK3NX

- Best Moon noise measured at  2.2dB

- Moon noise at time of sked =  2.0dB

Condx at  VK3NX  on 10/10/2014  @  time of  QSO

- Cloudy  low  level

- Humidity 74%

- Air temp 13 deg C

- Apparent Air temp =11 Deg

- Dew point 9.0 Deg C

- Pw =

Pics of VK3NX  equipment:

Various  screen  shots…..

VK3UM  Libration  Calculator