VK3NX – OK1KIR    QSO  16/11/2013  24 GHz  JT4

CW Contact also followed

19/11/2013

Equipment VK3NX:

- 2.4 m  Solid dish  (Andrews)

- 11.5 W Kuhne SSPA  (10 W at feed)  short  WR42  flexible W/G  to  W/G Switch (~0.5dB loss)

- Feed:  Super VE4MA  choke flange   with  Circular to rectangular transition (WR42)

- W/G switch:  “EVENTS HORIZON”

- LNA @ feed

  - Kuhne 1.5dB NF  27 db gain  +  DB6NT design (homebrew)  NF= Max 2.0dB  Gain =  11.5dB

  - Total  gain is 38.5dB in “feed box”

- LMR240  with Radiall 18GHz connectors  for  short  run  on Tx  and Rx  from “feed box” to  transverter

- Transverter = Thales  26 GHz  units  running  1296 MHz  IF.  Then  down-converted to  144MHz IF.   IF  driver  IC-746.  ( With  500 Hz  filter  used for CW  contact)

- Kuhne  24 GHz  W/G  filters  on  Rx and Tx  ports of Thales  unit.  (Absolutely necessary due to very   poor image rejection ~4dB only  with 1296 MHz  IF)

Perfomance:

- Best Moon noise measured at  2.2dB

- Moon noise at time of sked =  1.8dB

Condx on 18/11/2013:

- Light fog / precipitation in the air.

- Humidity ~90%

- PW= 17-20mm  during sked with OK1KIR  (as calculated by G3WDG)

SPECTRAN view of  1558z rcvd from OK1KIR

Same Signal (1558) in SpecJT

WSJT SCREEN of  1558

Spectran  view  screen of  1604z  single tone

Single tone Signal at  1604z  in  SpecJT

Screen Shot  of  1604z

On 16/11/2013