VK3NX –JA1WQF   JT4F QSO  REPORT

DATE : 7th Dec 2013  0940- 1000z

Wx at VK3NX  Approx 20 deg Celsius   69% humidity  No cloud No wind

Great circle Distance = 8212km  (WSJT figure)

Elevation @ V3NX @0930z = 40 degrees   @1000z= 35 degrees

Loc at  VK3NX QF21CT

Loc at JA1WQF  QM05AV

Equipment VK3NX:

- 2.4 m  Solid dish  (Andrews)

- 11.5 W Kuhne SSPA  (10 W at feed)  short  WR42  flexible W/G  to  W/G Switch (~0.5dB loss)

- Feed:  Super VE4MA  choke flange   with  Circular to rectangular transition (WR42)

- W/G switch:  “EVENTS HORIZON”

- LNA @ feed

  - Kuhne 1.5dB NF  27 db gain  +  DB6NT design (homebrew)  NF= Max 2.0dB  Gain =  11.5dB

  - Total  gain is 38.5dB in “feed box”

- LMR240  with Radiall 18GHz connectors  for  short  run  on Tx  and Rx  from “feed box” to  transverter

- Transverter = Thales  26 GHz  units  running  1296 MHz  IF.  Then  down-converted to  144MHz IF.   IF  driver  IC-746.  ( With  500 Hz  filter  used for CW  contact)

- Kuhne  24 GHz  W/G  filters  on  Rx and Tx  ports of Thales  unit.  (Absolutely necessary due to very   poor image rejection ~4dB only  with 1296 MHz  IF)

Perfomance:

- Best Moon noise measured at  2.2dB

- Moon noise at time of sked =  1.8dB

Equipment  @  JA1WQF

- 2.4m dish

- Power 10 W

WSJT  DECODED TEXT:

092800  0  -21  5.7  -31  22 *

092937  Transmitting: JT4F    JA1WQF VK3NX QF21

092946  Transmitting: JT4F    @1270

094800  7  -14  2.3 -317  96 *      VK3NX JA1WQF             1    0 E

094900  Transmitting: JT4F    JA1WQF VK3NX -14

095000  7  -14  2.3 -389 101 *      VK3NX JA1WQF             1    0 D

095000  1   1/1                     VK3NX JA1WQF             1    0

095200  4  -16  2.2 -396  90 *

095400  5  -15  2.6 -418  94 *

095600  5  -15  2.3 -365 101 *      R-17                     1    0 D

095700  Transmitting: JT4F    JA1WQF VK3NX RRR

095800  6  -14  2.3 -438 101 *      73                       1    0 D

095900  Transmitting: JT4F    @1700

100000  6  -14  2.5 -466  74 *

100000  1   6/6                     73                       1    0

100100  0  -21  1.2  -70   7 #

100100  1   6/6                     73                       1    0

WSJT  SCREEN  SHOTS:

0944z  JA1WQF 1270 Hz Tone

Frequency correction (netting)  by  VK3NX on  1270 Hz tone Rx from JA1WQF

First  message sent by  JA1WQF @ 0948

Decode of  first  Message.   (Bin Width of “E” indicated  by  program for decode)

Signal at  0956

Decode  @  0956

VK3UM  Libration Calculator on day of sked of sked

Screen shot  from  JA1WQF

Modified Thales xvtr at VK3NX

VK3NX 2.4m dish