VK3NX – LX1DB 24 GHz CW QSO  REPORT

1st  VK-LX  CW  QSO  on 24 GHz

DATE : 27th April 2014  0530-0550z

- Wx at VK3NX

  - 60% humidity

  - Light cloud

  - Windy  20-25 km/hr  gusts

- Wx at  LX1DB

- Temperature was 11°C and dew point 4°C so low humidity

- Spreading of signal 120Hz during QSO

Elevation @ V3NX

- @0530  = 9.2 Degrees  START  QSO

- @0550 = 5.5 degrees  END  QSO

Loc at  VK3NX QF21CT

Loc at LX1DB  JN49CO

Great circle Distance = 16324.3 km .. New  VK  and  world  CW  24GHz record.

Equipment VK3NX:

- 2.4 m  Solid dish  (Andrews)

- 11.5 W Kuhne SSPA  (10 W at feed)  short  WR42  flexible W/G  to  W/G Switch (~0.5dB loss)

- Feed:  Super VE4MA  choke flange   with  Circular to rectangular transition (WR42)

- W/G switch:  “EVENTS HORIZON”

- LNA @ feed

  - Kuhne 1.5dB NF  27 db gain  +  DB6NT design (homebrew)  NF= Max 2.0dB  Gain =  11.5dB

  - Total  gain is 38.5dB in “feed box”

- LMR240  with Radiall 18GHz connectors  for  short  run  on Tx  and Rx  from “feed box” to  transverter

- Transverter = Thales  26 GHz  units  running  1296 MHz  IF.  Then  down-converted to  144MHz IF.   IF  driver  IC-7700

- Kuhne  24 GHz  W/G  filters  on  Rx and Tx  ports of Thales  unit.  (Absolutely necessary due to very   poor image rejection ~4dB only  with 1296 MHz  IF)

Perfomance:

- Best Moon noise measured at  2.2dB

- Moon noise at time of sked =  1.8dB  and  falling!

Equipment at   LX1DB:

Moon noise 2,4db at time of  QSO

CS/G ratio 3,4db

Dish 3m

TX 42W at the feed

Solar noise: 15,db at flux 100

moon noise: 2,5db  at flux 10

INFORMATION @  Time of  QSO:

FROM  VK3UM  software:

Red =  VK3NX

Blue = LX1DB

Green  =  LX1DB  @  VK3NX

Signal width of  LX1DB  received at  VK3NX  approximately  125 Hz